# Analysis Notes

# Purpose

The purpose of this document is to capture observations made during the Kalman filter sensitivity study. The notes are intended as supporting material for a future report and are not intended to present final conclusions.

# Notes

## Train speed noise model

### Purpose

A white Gaussian noise model was considered unrealistic for train speed measurements because consecutive samples are independent. Real sensors normally exhibit correlated errors.

### Experiment

A low-pass filtered Gaussian noise model with a standard deviation of 2 % was applied to the TrainSpeed measurements. The low-pass filtering produces correlated measurement errors, representing a more realistic sensor behaviour than white Gaussian noise. This perturbation model was used in the subsequent tunnel experiments.

Figure 1. Low-pass filtered TrainSpeed perturbation used in the tunnel experiments.

### Observations

- The generated noise varies smoothly over time.

- No abrupt jumps are present.

- The signal resembles a slowly drifting sensor error.

### Open questions

## 60 s GPS outage

### Purpose

To study how the position estimate evolves during a prolonged GPS outage under realistic TrainSpeed measurements.

### Experiment

GPS measurements were disabled for 60 s while the TrainSpeed measurements remained available with the previously defined noise model. During the outage, the Kalman filter relied primarily on odometry. The resulting position estimate was compared with Euler and trapezoidal dead reckoning using the same TrainSpeed input.

Figure 2. Position error during a 60 s GPS outage compared with two dead reckoning methods using the same TrainSpeed input.

### Observations

- During the GPS outage all three estimates gradually diverge from the reference.

- After GPS becomes available again the estimates converge back towards the reference.

- The three methods exhibit similar overall behaviour, although none remains consistently closest to the reference throughout the experiment.

### Open questions

Why does the position estimate recover so rapidly after GPS becomes available?

## Train speed calibration error

### Purpose

To investigate the influence of a systematic TrainSpeed calibration error.

### Experiment

A constant calibration error of ±2 % was applied to the TrainSpeed measurements throughout the simulation. No other sensor perturbations were introduced, allowing the isolated effect of systematic odometry errors to be evaluated.

Figure 3. Applied TrainSpeed calibration errors (−2 % and +2 %).

### Observations

A constant calibration error produces a systematic position error.

The position error evolves differently from the GPS outage experiment.

### Open questions

How much of the accumulated error is compensated by the GPS updates?

## GPS delay

### Purpose

To investigate the influence of delayed GPS measurements.

### Experiment

The GPS measurements were delayed by 500 ms, 1000 ms and 2000 ms, while all other sensor inputs remained unchanged. GPS measurements remained continuously valid throughout the experiment. The purpose was to isolate the influence of sensor latency on the estimated position.

Figure 4. Position error for three GPS delay values (500 ms, 1000 ms and 2000 ms).

Figure 5. Relationship between delayed GPS speed and position error for the 2000 ms delay case.

### Observations

Position error increases approximately with GPS delay.

A distinct reduction in position error occurs when braking begins.

The rapid correction coincides with a large discrepancy between the delayed GPS speed and the actual train speed.

### Open questions

Is the rapid reduction caused by the Kalman filter weighting changing during braking, or is it a consequence of the delayed GPS measurements interacting with the train dynamics?

## GPS position noise

### Purpose

To investigate how noise in the GPS position measurements affects the estimated train position.

### Experiment

Independent Gaussian noise was added to the latitude and longitude measurements before they were processed by the Kalman filter. Two noise levels, corresponding to standard deviations of 1 m and 10 m, were evaluated.

Figure 6 Position error for GPS position noise with standard deviations of 1 m and 10 m.

### Observations

GPS position noise is introduced by perturbing the latitude and longitude measurements before they are processed by the Kalman filter.

Increasing the GPS position noise standard deviation from 1 m to 10 m increases the variation of the estimated position error.

No obvious systematic bias is observed in either experiment.

### Open questions

## GPS speed measurement noise

### Purpose

To investigate how random noise in the GPS speed measurements affects the estimated train position.

### Experiment

Low-pass filtered Gaussian noise with standard deviations of 1 % and 5 % was applied to the GPS speed measurements. GPS position measurements remained unperturbed, allowing the influence of GPS speed noise to be studied in isolation.

Figure 7. Position error for GPS speed measurement noise with standard deviations of 1 % and 5 %.

### Observations

Increasing the GPS speed noise level from 1 % to 5 % produces only a modest increase in the position error.

The overall behaviour of the estimated position remains similar for both experiments.

No systematic drift is observed.

### Open questions

Why is the estimated position considerably less sensitive to GPS speed noise than to GPS position noise?

## Combined GPS outage and TrainSpeed calibration error

### Purpose

To investigate how the Kalman filter behaves when several realistic sensor disturbances are present simultaneously.

### Experiment

The experiment combines a 60 s GPS outage with a constant +5 % TrainSpeed calibration error. All other conditions were identical to the reference tunnel experiment. The objective was to evaluate how simultaneous sensor imperfections affect the estimated position.

Figure 8. Comparison of the reference tunnel scenario and the same scenario with an additional 5 % TrainSpeed calibration error.

### Observations

Adding a 5 % TrainSpeed calibration error dramatically increases the accumulated position error during the GPS outage.

Without calibration error, the position estimate remains close to the reference throughout the experiment.

The position estimate rapidly recovers once GPS measurements become available again.

### Open questions

Why does a constant TrainSpeed calibration error dominate the position estimate during the GPS outage?

# Overall observations

Random sensor noise generally produces bounded, noise-like position errors.

Systematic sensor errors tend to produce accumulated, drift-like position errors.

Sensor latency combines both behaviours and exhibits additional transient effects during braking.